Express Powerr Solutions (M) Bhd

Smart Fleet & Telematics Architecture

Technical Architecture Document

Version 1.0

Date: 23 July 2026

Classification: Confidential

# Table of Contents

1.  System Context & Overview

2.  Component Architecture

3.  Data Architecture

4.  Integration Architecture (ERP Staging Pattern)

5.  API Specifications

6.  IoT & Hardware Architecture

7.  Deployment Architecture

8.  Security Architecture

Appendix A — Glossary of Terms

Document Classification: Confidential — Express Powerr Solutions (M) Bhd

# 1. System Context & Overview

## 1.1 Business Context

Express Powerr Solutions (M) Bhd (Bursa: 0370) operates a fleet of over 115 generator units (low to high voltage) supported by heavy transport vehicles including prime movers, crane trucks, low-loaders, and service vans. Their primary operational base is the Klang depot. The company serves critical utility contracts with Tenaga Nasional Berhad (TNB) and Sabah Electricity Sdn Bhd (SESB) for emergency power cuts, planned shutdowns, and standby generation.

Diesel fuel represents approximately 72%–78% of direct operational expenses. The company is staffed with certified chargemen and engine drivers who deploy with generator equipment to client sites. On the financial side, Express Powerr uses SQL Account / ERP for Delivery Orders (DOs), e-Invoicing, SST reporting, and LHDN compliance.

## 1.2 System Context (C4 Model)

The diagram below defines the outer boundary of the Smart Fleet system — the users and external systems it interacts with.

Mermaid-compatible C4 Context Diagram:

┌─────────────────────────────────────────────────────────────────────────────────┐ │ C4 SYSTEM CONTEXT DIAGRAM │ │ │ │ ┌──────────────┐ ┌───────────────────────────────────────┐ │ │ │ TNB / SESB │ │ Express Powerr │ │ │ │ (External │──┬───>│ [Smart Fleet System] │ │ │ │ Client) │ │ │ ┌─────────────────────────────────┐ │ │ │ └──────────────┘ │ │ │ Fleet Cloud (GPS, Fuel, │ │ │ │ │ │ │ Telematics, Dispatch) │ │ │ │ ┌──────────────┐ │ │ └───────────┬─────────────────────┘ │ │ │ │ Industrial │ │ │ │ │ │ │ │ Clients │──┤ │ ▼ │ │ │ └──────────────┘ │ │ ┌──────────────────────────┐ │ │ │ │ │ │ Intermediate Staging DB │ │ │ │ ┌──────────────┐ │ │ │ (Validation & Error │ │ │ │ │ LHDN │ │ │ │ Log) │ │ │ │ │ (Tax Auth) │ │ │ └───────────┬──────────────┘ │ │ │ └──────────────┘ │ │ │ │ │ │ │ │ ▼ │ │ │ ┌──────────────┐ │ │ ┌──────────────────────────┐ │ │ │ │ Fuel Card │ │ │ │ Live SQL Account/ERP DB │ │ │ │ │ Providers │──┤ │ │ (DOs, Ledger, Assets) │ │ │ │ │(Shell/Petronas│ │ │ └──────────────────────────┘ │ │ │ │ /BHPetrol) │ │ │ │ │ │ └──────────────┘ │ └───────────────────────────────────────┘ │ │ │ │ │ ┌──────────────┐ │ │ │ │ Drivers / │──┤ │ │ │ Chargemen │ │ │ │ └──────────────┘ │ │ │ │ │ External Users System Under Design External Systems │ └─────────────────────────────────────────────────────────────────────────────────┘

## 1.3 Key Actors & Roles

| Actor | Role | Primary Interface |
| --- | --- | --- |
| Dispatcher | Assigns jobs, monitors fleet, handles emergency deployment | Admin Web Portal |
| Driver / Engine Driver | Operates transport vehicle, accepts jobs, submits POD | Driver Mobile App |
| Chargeman | Supervises generator setup on-site, certifies electrical safety | Driver Mobile App |
| Finance Team | Reconciles fuel claims, generates DOs, processes e-Invoices | SQL Account ERP (existing) |
| Fleet Manager | Monitors fuel consumption, maintenance schedules, asset utilisation | Admin Web Portal |
| TNB / SESB | Initiates emergency call-outs, requires SLA compliance proof | Phone / Email (inbound) |

## 1.4 Key Architectural Principles

No Direct Schema Writes to ERP: The Fleet Cloud never executes direct SQL INSERT/UPDATE/DELETE on the live SQL Account database. All data passes through the staging layer.

Staging Buffer Layer: Data from the fleet system lands in intermediate staging tables. The ERP imports only after field-level validation passes.

Cellular-First Telemetry: OBD-II/CAN-bus devices and BLE sensors communicate over 4G/5G cellular networks directly to the cloud — zero load on Express Powerr's local IT infrastructure.

API-First Integration: All cross-system communication uses protected REST API endpoints or scheduled webhooks. Failed payloads are logged in isolation without affecting ERP ledgers.

Offline-Resilient Mobile: The Driver Mobile App cores job data locally and syncs when connectivity is restored — critical for rural TNB/SESB sites with poor coverage.

# 2. Component Architecture

## 2.1 Container Diagram (C4)

The Smart Fleet system is composed of the following major containers running inside the Fleet Cloud boundary plus on-device components.

Mermaid-compatible C4 Container Diagram:

┌──────────────────────────────────────────────────────────────────────────────────┐ │ C4 CONTAINER DIAGRAM — SMART FLEET SYSTEM │ │ │ │ ┌─────────────────────────────────────────────────────────────────┐ │ │ │ FLEET CLOUD (Azure / AWS) │ │ │ │ │ │ │ │ ┌──────────────┐ ┌──────────────┐ ┌──────────────────────┐ │ │ │ │ │ API │ │ WebSocket │ │ Job Dispatcher │ │ │ │ │ │ Gateway │ │ Server │ │ Engine │ │ │ │ │ │ (REST/gRPC) │ │ (Real-time) │ │ (SLA Scheduler) │ │ │ │ │ └──────┬───────┘ └──────┬───────┘ └──────────┬───────────┘ │ │ │ │ │ │ │ │ │ │ │ ▼ ▼ ▼ │ │ │ │ ┌──────────────────────────────────────────────────────┐ │ │ │ │ │ Core Services Layer │ │ │ │ │ │ ┌──────────┐ ┌─────────┐ ┌────────┐ ┌───────────┐ │ │ │ │ │ │ │ Fleet │ │ Fuel │ │ Asset │ │ POD / │ │ │ │ │ │ │ │ Tracking │ │ Manager │ │ Binding│ │ Document │ │ │ │ │ │ │ └──────────┘ └─────────┘ └────────┘ └───────────┘ │ │ │ │ │ │ ┌──────────┐ ┌─────────┐ ┌────────┐ ┌───────────┐ │ │ │ │ │ │ │Geofence │ │ SLA │ │ Alert │ │ ERP Sync │ │ │ │ │ │ │ │ Engine │ │ Monitor │ │ Engine │ │ Bridge │ │ │ │ │ │ │ └──────────┘ └─────────┘ └────────┘ └───────────┘ │ │ │ │ │ └──────────────────────────────────────────────────────┘ │ │ │ │ │ │ │ │ │ ▼ │ │ │ │ ┌──────────────────────────────────────────────────────┐ │ │ │ │ │ Data Layer │ │ │ │ │ │ ┌────────────┐ ┌────────────┐ ┌──────────────────┐ │ │ │ │ │ │ │ Fleet DB │ │ Staging DB │ │ File / Object │ │ │ │ │ │ │ │ (Timescale)│ │ (Postgres) │ │ Store (Photos, │ │ │ │ │ │ │ │ │ │ │ │ Signatures) │ │ │ │ │ │ │ └────────────┘ └────────────┘ └──────────────────┘ │ │ │ │ │ └──────────────────────────────────────────────────────┘ │ │ │ └─────────────────────────────────────────────────────────────────┘ │ │ │ │ ┌────────────────────┐ ┌────────────────────┐ ┌────────────────────────┐ │ │ │ Driver Mobile App │ │ Admin Web Portal │ │ IoT Device Layer │ │ │ │ (Flutter / React │ │ (React.js) │ │ ┌──────────────────┐ │ │ │ │ Native) │ │ │ │ │ OBD-II / CAN-bus │ │ │ │ │ • Job List │ │ • Dashboard │ │ │ (Vehicle │ │ │ │ │ • Turn-by-turn │ │ • Fleet Map │ │ │ Telemetry) │ │ │ │ │ • POD Capture │ │ • Fuel Analytics │ │ └──────────────────┘ │ │ │ │ • Offline Queue │ │ • SLA Reports │ │ ┌──────────────────┐ │ │ │ └────────────────────┘ └────────────────────┘ │ │ BLE Beacons │ │ │ │ │ │ (Generator Tags) │ │ │ │ ┌────────────────────┐ │ └──────────────────┘ │ │ │ │ SQL Account ERP │ │ ┌──────────────────┐ │ │ │ │ (Existing System) │ │ │ Capacitance │ │ │ │ │ • DOs │<── Staging Sync ────┘ │ │ Fuel Probes │ │ │ │ │ • Ledger │ │ └──────────────────┘ │ │ │ │ • e-Invoice │ └────────────────────────┘ │ │ │ • SST / LHDN │ │ │ └────────────────────┘ │ └───────────────────────────────────────────────────────────────────────────────────┘

## 2.2 Component Descriptions

API Gateway: Single entry point for all client requests. Handles authentication (OAuth2/JWT), rate limiting, request validation, and routes traffic to the appropriate core service. Exposes RESTful endpoints for the Admin Portal, Driver App, and external webhook consumers.

WebSocket Server: Maintains persistent connections with the Driver Mobile App for real-time dispatch notifications, geofence breach alerts, and job status updates. Uses a topic-based pub/sub pattern.

Job Dispatcher Engine: SLA-aware scheduling engine that matches incoming service requests (emergency / planned shutdown) to available vehicles, generators, and crew. Factors in current GPS positions, ETA calculations, and crew certification (chargeman vs. engine driver).

Fleet Tracking Service: Ingests GPS telemetry from OBD-II/CAN-bus devices at configurable intervals (1–60 sec). Stores time-series data in a TimescaleDB instance. Provides reverse-geocoding, trail history, and trip segmentation.

Fuel Manager: Aggregates data from capacitance fuel probes (tank level) and fuel card transactions. Detects anomalies (sudden drops, off-hours refueling, GPS-card mismatch). Generates fuel consumption reports and alerts for suspected theft.

Asset Binding Service: Processes BLE beacon scan reports from vehicle-mounted scanners. Determines which generator unit(s) are currently loaded on which truck by matching beacon IDs against the asset registry. Publishes binding change events.

POD / Document Service: Accepts digital signature captures and photo uploads from the Driver App. Assembles a proof-of-delivery package and triggers a staging DO record for ERP ingestion.

Geofence Engine: Evaluates real-time vehicle positions against predefined geofences (client sites, Klang depot, fuel stations). Fires entry/exit/dwell events consumed by the SLA Monitor and Dispatcher.

SLA Monitor: Tracks contractual SLA metrics for TNB/SESB: response time (minutes from call to site arrival), generator online time, site departure. Generates compliance reports and breach alerts.

ERP Sync Bridge: The only component that writes to the Intermediate Staging DB. Transforms fleet events into ERP-compatible payloads (fuel vouchers, DO drafts, asset location updates). The staging DB is read by a scheduled job within Express Powerr's network to update the live SQL Account DB.

# 3. Data Architecture

## 3.1 Entity Relationship Overview

The Fleet Cloud maintains its own operational database independent of the SQL Account ERP. The primary entities are described below. The staging schema (Section 4) mirrors a subset of these for ERP synchronisation.

Entity Relationship Diagram:

┌────────────────────────────────────────────────────────────────────────────────────┐ │ PHYSICAL ERD (Fleet Cloud DB) │ │ │ │ ┌──────────────┐ ┌──────────────────┐ ┌───────────────────┐ │ │ │ vehicles │ │ generators │ │ employees │ │ │ ├──────────────┤ ├──────────────────┤ ├───────────────────┤ │ │ │PK vehicle_id │ │PK generator_id │ │PK employee_id │ │ │ │ plate_no │ │ serial_no │ │ name │ │ │ │ vehicle_type │ │ voltage_rating │ │ role (driver/ │ │ │ │ make_model │ │ power_kva │ │ chargeman) │ │ │ │ year │ 1 │ ble_beacon_id │ │ cert_number │ │ │ │ can_bus_ │───────│ status (avail/ │ │ phone │ │ │ │ supported │ │ deployed) │ │ ──────────────────│ │ │ │ tank_cap_l │ │ ─────────────────│ │ M │ │ │ │ ──────────── │ │ M │ │ │ │ │ │ │ M │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ └──┼───────────┘ └──┼───────────────┘ └──┼────────────────┘ │ │ │ │ │ │ │ │ ┌───────────┼───────────────────────────┼──────────┐ │ │ │ │ │ │ │ │ │ │ │ ┌─────┴──────────────────────────────┐ │ │ │ │ │ │ trip_jobs │ │ │ │ │ │ ├────────────────────────────────────┤ │ │ │ │ │ │PK trip_id │ │ │ │ │ │ │FK vehicle_id │<─────┘ │ │ │ │ │FK generator_id │<────────────┐ │ │ │ │ │FK driver_id │ │ │ │ │ │ │FK chargeman_id │ │ │ │ │ │ │ client_name │ │ │ │ │ │ │ client_site │ │ │ │ │ │ │ job_type (emergency/planned) │ │ │ │ │ │ │ sla_timer_minutes │ │ │ │ │ │ │ status │ │ │ │ │ │ │ dispatched_at │ │ │ │ │ │ │ site_arrival_at │ │ │ │ │ │ │ completed_at │ │ │ │ │ │ └────────────────────────────────────┘ │ │ │ │ │ │ │ │ │ │ ┌──────────────────────┐ ┌──────────────────┐ │ │ │ │ │ │ fuel_logs │ │ pod_records │ │ │ │ │ │ ├──────────────────────┤ ├──────────────────┤ │ │ │ │ └────>│PK fuel_log_id │ │PK pod_id │ │ │ │ │ │FK trip_id │<────│FK trip_id │ │ │ │ │ │FK vehicle_id │ │ signature_data │ │ │ │ │ │ litres │ │ photo_urls[] │ │ │ │ │ │ fuel_type │ │ notes │ │ │ │ │ │ gps_lat/lon │ │ submitted_at │ └─────────────┘ │ │ │ probe_reading_pct │ └──────────────────┘ │ │ │ card_transaction_ref │ │ │ │ timestamp │ ┌──────────────────────┐ │ │ └──────────────────────┘ │ geofence_events │ │ │ ├──────────────────────┤ │ │ ┌──────────────────────┐ │PK event_id │ │ │ │ ble_scan_events │ │FK vehicle_id │ │ │ ├──────────────────────┤ │ geofence_name │ │ │ │PK scan_id │ │ event_type │ │ │ │FK vehicle_id │ │ (entry/exit/dwell) │ │ │ │ beacon_id │ │ timestamp │ │ │ │ rssi │ └──────────────────────┘ │ │ │ scanned_at │ │ │ └──────────────────────┘ └────────────────────────────────────────────────────────────────────────────────────┘

## 3.2 Core Entity Definitions

| Entity | Description | Key Fields | ERP Sync? |
| --- | --- | --- | --- |
| vehicles | Prime movers, crane trucks, low-loaders, service vans | plate_no, vehicle_type, can_bus_supported, tank_cap_l | Yes — Asset module |
| generators | 115+ gen sets, LV to HV, each with optional BLE beacon | serial_no, voltage_rating, power_kva, ble_beacon_id, status | Yes — Asset module |
| employees | Drivers, engine drivers, chargemen with certifications | name, role, cert_number, phone | No (HR kept in ERP) |
| trip_jobs | Dispatch record linking vehicle + gen + crew to a service call | client_name, site, job_type, sla_timer_minutes, status, timestamps | Yes → Draft DO |
| fuel_logs | Fuel events from probe readings and card transactions | litres, gps_lat/lon, probe_reading_pct, card_transaction_ref | Yes → Purchase Voucher |
| pod_records | Proof of delivery: signatures, photos, notes | signature_data, photo_urls[], submitted_at | Yes → DO attachment |
| ble_scan_events | BLE beacon sightings from vehicle scanners | vehicle_id, beacon_id, rssi, scanned_at | No (operational) |
| geofence_events | Vehicle entry/exit at client sites, depot, fuel stations | vehicle_id, geofence_name, event_type, timestamp | No (operational) |

## 3.3 BLE Beacon Binding Logic

Each generator unit can be fitted with a BLE beacon broadcasting a unique UUID at 1 Hz. Every transport vehicle carries a BLE scanner (Raspberry Pi / ESP32) that reports all visible beacon IDs with RSSI values every 5 seconds.

The Asset Binding Service resolves the current binding using a simple heuristic: the beacon with the highest average RSSI over the last 60-second window that falls within expected range (–40 dBm to –75 dBm) is considered 'loaded' on that vehicle. When a beacon disappears for more than 120 seconds, a binding-change event is published (unloaded from vehicle X, available for next assignment).

# 4. Integration Architecture (ERP Staging Pattern)

## 4.1 Rationale

Express Powerr's finance team relies on SQL Account (or a comparable SQL Server-based ERP) for DO issuance, e-Invoicing, SST reporting, and LHDN compliance. The primary concern when introducing a smart fleet system is the risk of corrupting database tables, locking schemas, or disrupting e-Invoice synchronisation.

The architecture below guarantees that the Fleet Cloud never writes directly to the live ERP database. All data destined for the ERP flows through an isolated Intermediate Staging Database.

ERP Staging Pattern — Data Flow:

┌───────────────────────────────────────────────────────────────────────────────────┐ │ ERP STAGING PATTERN — DATA FLOW │ │ │ │ ┌───────────────────────────┐ │ │ │ Fleet Cloud │ │ │ │ (GPS, Fuel, Telematics) │ │ │ │ │ │ │ │ ┌─────────────────────┐ │ REST API (HTTPS, JWT) │ │ │ │ ERP Sync Bridge │──┼─────────────────────────────────────────────────┐ │ │ │ │ Service │ │ │ │ │ │ └─────────────────────┘ │ │ │ │ └───────────────────────────┘ │ │ │ │ │ │ ▼ │ │ ┌──────────────────────────────────┐ │ │ │ Intermediate Staging DB │ │ │ │ (PostgreSQL, isolated schema) │ │ │ │ │ │ │ │ ┌────────────────────────────┐ │ │ │ │ │ staging_fuel_logs │ │ │ │ │ │ • id (PK) │ │ │ │ │ │ • fleet_cloud_id │ │ │ │ │ │ • vehicle_id │ │ │ │ │ │ • litres │ │ │ │ │ │ • gps_coords │ │ │ │ │ │ • card_ref │ │ │ │ │ │ • timestamp │ │ │ │ │ │ • status (pending/ │ │ │ │ │ │ validated/ │ │ │ │ │ │ error) │ │ │ │ │ │ • error_message │ │ │ │ │ └────────────────────────────┘ │ │ │ │ ┌────────────────────────────┐ │ │ │ │ │ staging_delivery_orders │ │ │ │ │ │ • id (PK) │ │ │ │ │ │ • fleet_cloud_id │ │ │ │ │ │ • trip_id │ │ │ │ │ │ • client_name │ │ │ │ │ │ • site_address │ │ │ │ │ │ • generator_serial │ │ │ │ │ │ • signature_b64 │ │ │ │ │ │ • photo_urls[] │ │ │ │ │ │ • status │ │ │ │ │ │ • error_message │ │ │ │ │ └────────────────────────────┘ │ │ │ │ ┌────────────────────────────┐ │ │ │ │ │ staging_asset_updates │ │ │ │ │ │ • asset_type │ │ │ │ │ │ • asset_id │ │ │ │ │ │ • gps_coords │ │ │ │ │ │ • last_known_location │ │ │ │ │ │ • status │ │ │ │ │ └────────────────────────────┘ │ │ │ └──────────────────────────────────┘ │ │ │ │ │ Scheduled Job (SQL Agent / │ │ │ Windows Task Scheduler) │ │ │ Runs every 5 minutes │ │ ▼ │ │ ┌──────────────────────────────────┐ │ │ │ VALIDATION LAYER │ │ │ │ (within ERP network) │ │ │ │ │ │ │ │ ✓ Required fields present │ │ │ │ ✓ Vehicle/gen exists in ERP │ │ │ │ ✓ Fuel card ref is valid │ │ │ │ ✓ GPS coords within Malaysia │ │ │ │ ✓ Timestamp not in future │ │ │ │ │ │ │ │ If FAIL → status='error', │ │ │ │ log error_message │ │ │ └──────────────────────────────────┘ │ │ │ │ │ ▼ │ │ ┌──────────────────────────────────┐ │ │ │ Live SQL Account / ERP DB │ │ │ │ │ │ │ │ • DOs → Delivery Order table │ │ │ │ • Fuel → Purchase Voucher/GL │ │ │ │ • Assets → Asset Location │ │ │ │ • e-Invoice trigger (LHDN) │ │ │ └──────────────────────────────────┘ │ └───────────────────────────────────────────────────────────────────────────────────────────┘

## 4.2 Staging Table Definitions

| Staging Table | Source Event | Target ERP Module | Validation Rules |
| --- | --- | --- | --- |
| staging_fuel_logs | Fuel card + probe data | Purchase Voucher / GL | vehicle_id exists, litres > 0, timestamp ±5 min |
| staging_delivery_orders | Trip completion + POD | DO module → e-Invoice | client_name required, signature present, generator_serial valid |
| staging_asset_updates | Geofence + BLE binding change | Asset Management | asset_type IN ('vehicle','generator'), coords within Malaysia |
| staging_mileage | Odometer reading event | PO for servicing | vehicle_id exists, reading > last_reading |

## 4.3 Error Handling & Retry

When a staging record fails validation, it is not removed. Its status is set to 'error' and the error_message column captures the reason. The Fleet Cloud Dashboard exposes a dedicated 'ERP Sync Errors' view where the finance team can review, correct, and manually retry individual records. An automatic retry mechanism (3 attempts with exponential backoff) runs for transient failures (network timeout, deadlock). Persistent validation errors require human intervention.

## 4.4 Sequence: End-to-End Trip Sync

End-to-End Trip → DO Sync Sequence:

┌────────────────────────────────────────────────────────────────────────────────────┐ │ END-TO-END TRIP → DO SYNC SEQUENCE │ │ │ │ Driver App Fleet Cloud Staging DB ERP (SQL Acct) Finance │ │ │ │ │ │ │ │ │ │ Job Accept │ │ │ │ │ │ │─────────────────>│ │ │ │ │ │ │ │ │ │ │ │ │ │ GPS Telemetry │ │ │ │ │ │ │─────────────────>│ │ │ │ │ │ │ (continuous) │ │ │ │ │ │ │ │ │ │ │ │ │ │ Site Arrival │ │ │ │ │ │ │─────────────────>│ Geofence Event │ │ │ │ │ │ │──────────────────>│ │ │ │ │ │ │ │ │ │ │ │ │ POD Submit │ │ │ │ │ │ │ (sign+photo) │ │ │ │ │ │ │─────────────────>│ │ │ │ │ │ │ │ 1. Validate POD │ │ │ │ │ │ │ 2. Create DO │ │ │ │ │ │ │ staging record │ │ │ │ │ │ │──────────────────>│ │ │ │ │ │ │ │ │ │ │ │ │ │ │ Scheduled Job │ │ │ │ │ │ │ (every 5 min) │ │ │ │ │ │ │──────────────────>│ │ │ │ │ │ │ │ │ │ │ │ │ │ │ Validate & │ │ │ │ │ │ │ Insert DO │ │ │ │ │ │ │──────────────│─────>│ │ │ │ │ │ │ │ │ │ │ │ ACK / Error │ │ │ │ │ │ │<──────────────────│ │ │ │ │ │ │ │ │ │ │ │ Sync Status │ │ │ │ │ │ │<─────────────────│ │ │ │ │ └────────────────────────────────────────────────────────────────────────────────────┘

# 5. API Specifications

## 5.1 API Gateway Base URL

Production: https://fleet-api.expresspowerr.com.my/v1

Staging:    https://fleet-api-staging.expresspowerr.com.my/v1

## 5.2 Authentication

All API requests require a Bearer JWT token obtained via the OAuth2 Client Credentials flow (for service-to-service) or Authorization Code flow (for user-facing apps). Tokens expire after 60 minutes and must be refreshed.

## 5.3 Core Endpoints

### Fleet & Tracking

| Method | Path | Description | Auth |
| --- | --- | --- | --- |
| GET | /vehicles | List all vehicles with current status | fleet:read |
| GET | /vehicles/{id} | Vehicle details + latest telemetry | fleet:read |
| GET | /vehicles/{id}/telemetry | Time-series telemetry (query params: from, to, interval) | fleet:read |
| GET | /generators | List all generator units with BLE beacon bindings | asset:read |
| PUT | /generators/{id}/beacon | Assign/update BLE beacon ID to a generator | asset:write |
| GET | /bindings/current | Current vehicle↔generator binding snapshot | asset:read |

### Dispatch & Jobs

| Method | Path | Description | Auth |
| --- | --- | --- | --- |
| POST | /jobs | Create a new dispatch job (emergency or planned) | dispatch:write |
| GET | /jobs | List jobs with filters (status, date, client) | dispatch:read |
| GET | /jobs/{id} | Job details with timeline | dispatch:read |
| PATCH | /jobs/{id}/assign | Assign vehicle + generator + crew to job | dispatch:write |
| POST | /jobs/{id}/dispatch | Send dispatch notification to driver app | dispatch:write |
| POST | /jobs/{id}/pod | Submit proof of delivery (signature + photos) | pod:write |

### Fuel Management

| Method | Path | Description | Auth |
| --- | --- | --- | --- |
| POST | /fuel/logs | Ingest a fuel event (probe and/or card) | fuel:write |
| GET | /fuel/logs | Query fuel logs (vehicle, date range) | fuel:read |
| GET | /fuel/anomalies | List flagged anomalies (theft alerts, mismatches) | fuel:read |
| POST | /fuel/card/import | Import fuel card statement CSV | fuel:write |

### Geofence & SLA

| Method | Path | Description | Auth |
| --- | --- | --- | --- |
| POST | /geofences | Create a geofence polygon (client site, depot, etc.) | geo:write |
| GET | /geofences | List geofences | geo:read |
| GET | /geofences/{id}/events | Events for a specific geofence | geo:read |
| GET | /sla/reports | SLA compliance report (query: client, month) | sla:read |

### ERP Sync (Admin)

| Method | Path | Description | Auth |
| --- | --- | --- | --- |
| GET | /sync/staging-queue | List pending staging records with status | sync:read |
| POST | /sync/retry/{id} | Manually retry a failed staging record | sync:write |
| GET | /sync/errors | List all records in 'error' status with messages | sync:read |
| GET | /sync/logs | Sync job execution history | sync:read |

## 5.4 Webhook Events

The system can push events to external consumers (e.g., ERP import trigger, SMS gateway) via registered webhook URLs. Each webhook payload is signed with an HMAC-SHA256 secret.

| Event Type | Trigger | Payload Summary |
| --- | --- | --- |
| job.dispatched | Job assigned and pushed to driver app | job_id, vehicle_id, generator_id, crew, site_coords |
| geofence.entry | Vehicle enters a geofence | vehicle_id, geofence_name, timestamp, coords |
| geofence.exit | Vehicle exits a geofence | vehicle_id, geofence_name, timestamp, duration_min |
| fuel.anomaly | Sudden fuel drop or GPS mismatch detected | vehicle_id, litres_delta, gps_match (bool), confidence |
| binding.changed | BLE beacon binding between vehicle and generator changed | vehicle_id, generator_id, action (load/unload), timestamp |
| sync.record.ready | Staging record validated and ready for ERP import | staging_table, record_id, target_module |
| sync.record.error | Staging record failed validation | staging_table, record_id, error_message |

# 6. IoT & Hardware Architecture

## 6.1 On-Vehicle Telemetry Device

Each transport vehicle is equipped with an OBD-II/CAN-bus telematics gateway that supports the J1939 protocol (standard for heavy-duty vehicles). This device extracts:

Engine RPM and load percentage

Fuel level from the vehicle's OEM tank sender (via CAN-bus)

Engine hours and odometer reading

Diagnostic Trouble Codes (DTCs) for preventive maintenance

GPS position (internal GNSS receiver)

The telematics gateway transmits data over 4G LTE (fallback to 3G/2G where LTE is unavailable, common in rural Sabah/Sarawak sites). Data transmission interval is configurable: 1 second during active dispatch, 60 seconds during idle/standby.

## 6.2 Fuel Level Monitoring

For fuel theft detection and accurate consumption tracking, each vehicle tank is fitted with a capacitance-type fuel level sensor (submersible probe). The probe communicates with the telematics gateway via RS232/RS485 or analog 0–5V input.

The Fuel Manager service in the cloud compares probe readings against card refuel events. If the probe reports a drop > 10% without a corresponding ignition-off event or card refuel within a 5-minute window, a theft alert is raised.

## 6.3 BLE Beacon System for Generator Binding

BLE Beacon Binding Flow:

┌────────────────────────────────────────────────────────────────────────────────────┐ │ BLE BEACON — GENERATOR BINDING FLOW │ │ │ │ Generator Unit A Transport Truck Fleet Cloud │ │ (BLE Beacon Tag) (BLE Scanner) │ │ │ │ ┌────────────────┐ ┌──────────────────────┐ ┌──────────────┐ │ │ │ Beacon ID: │ Advertising │ BLE Scanner Module │ │ Asset │ │ │ │ GEN-BEACON-A001 │ (1 Hz) │ (ESP32 / RPi) │ │ Binding │ │ │ │ │────────────────>│ │ │ Service │ │ │ │ Broadcast UUID │ │ Collects all visible│ │ │ │ │ │ + battery level │ │ beacon IDs + RSSI │ │ │ │ │ └────────────────┘ │ every 5 seconds │ │ │ │ │ │ │ │ │ │ │ │ Bundles scans into │ │ │ │ │ │ batch report: │ REST │ │ │ │ │ [ │ POST │ │ │ │ │ {beacon: A001, │─────────>│ Resolve by │ │ │ │ rssi: -58}, │ │ highest avg │ │ │ │ {beacon: B002, │ │ RSSI over │ │ │ │ rssi: -82} │ │ 60s window │ │ │ │ ] │ │ │ │ │ │ │ │ Publish │ │ │ │ │ │ binding.changed event │ │ │ │ │ │ │ │ Generator Unit B │ │ │ Update │ │ │ (not shown) │ │ │ generators. │ │ │ If beacon disappears >120s │ │ │ status & │ │ │ → binding marked unloaded │ │ │ vehicle_id │ │ │ └──────────────────────┘ └──────────────┘ │ └────────────────────────────────────────────────────────────────────────────────────────────┘

## 6.4 Hardware Bill of Materials (Per Vehicle)

| Component | Purpose | Interface | Estimated Cost (MYR) |
| --- | --- | --- | --- |
| OBD-II/CAN-bus Telematics Gateway | Engine telemetry + GPS + cellular | J1939 CAN-bus, 4G LTE, GNSS | 800–1,200 |
| Capacitance Fuel Level Probe | Accurate tank level measurement | RS232/0–5V analog | 300–600 |
| BLE Scanner Module (ESP32) | Detect generator BLE beacons | Bluetooth 5.0, Wi-Fi (local config) | 80–150 |
| BLE Beacon Tag (per generator) | Generator identity broadcast | BLE 5.0, CR2032 battery (1yr life) | 20–40 |

## 6.5 Cellular Connectivity & Data Usage

Each telematics gateway uses a SIM card (CelcomDigi / Maxis / UMobile multi-network) with a pooled data plan. Estimated data usage per vehicle per month:

| Transmission Mode | Interval | Payload per Message | Monthly Data (est.) |
| --- | --- | --- | --- |
| Active Dispatch | 1 sec | ~200 bytes (GPS + CAN + probe) | ~500 MB |
| Standby / Idle | 60 sec | ~150 bytes | ~6.5 MB |
| BLE Scan Reports | 5 sec | ~100 bytes (batch of up to 10 beacons) | ~50 MB |

# 7. Deployment Architecture

## 7.1 Cloud Topology

The Fleet Cloud runs on Azure (alternatively AWS) within the Southeast Asia region (Singapore primary, Malaysia secondary for data residency requirements).

Cloud Deployment Topology:

┌──────────────────────────────────────────────────────────────────────────────────────┐ │ CLOUD DEPLOYMENT TOPOLOGY │ │ │ │ ┌───────────────────── Internet ──────────────────────┐ │ │ │ │ │ │ ▼ │ │ │ ┌──────────────────────────────────────────────┐ │ │ │ │ Azure Front Door / AWS CloudFront │ │ │ │ │ (CDN + WAF + DDoS Protection) │ │ │ │ └─────────────────────┬────────────────────────┘ │ │ │ │ │ │ │ ▼ │ │ │ ┌──────────────────────────────────────────────┐ │ │ │ │ Application Load Balancer (ALB/ALB) │ │ │ │ │ (SSL termination, routing) │ │ │ │ └──────┬──────────────┬──────────────┬──────────┘ │ │ │ │ │ │ │ │ │ ▼ ▼ ▼ │ │ │ ┌──────────┐ ┌──────────┐ ┌──────────┐ │ │ │ │ API │ │ API │ │ API │ Auto-Scale │ │ │ │ Server │ │ Server │ │ Server │ (min:2, │ │ │ │ (Node/ │ │ (Node/ │ │ (Node/ │ max:8) │ │ │ │ Go) │ │ Go) │ │ Go) │ │ │ │ └────┬─────┘ └────┬─────┘ └────┬─────┘ │ │ │ │ │ │ │ │ │ └──────────────┼──────────────┘ │ │ │ │ │ │ │ ▼ │ │ │ ┌──────────────────────────────────────────────┐ │ │ │ │ Azure Kubernetes Service (AKS) │ │ │ │ │ or AWS ECS / EKS │ │ │ │ │ │ │ │ │ │ ┌─────────┐ ┌─────────┐ ┌─────────────┐ │ │ │ │ │ │Fleet │ │Fuel │ │Asset │ │ │ │ │ │ │Tracking │ │Manager │ │Binding │ │ │ │ │ │ │(Pod) │ │(Pod) │ │(Pod) │ │ │ │ │ │ └─────────┘ └─────────┘ └─────────────┘ │ │ │ │ │ ┌─────────┐ ┌─────────┐ ┌─────────────┐ │ │ │ │ │ │Geofence │ │SLA │ │ERP Sync │ │ │ │ │ │ │(Pod) │ │Monitor │ │Bridge (Pod) │ │ │ │ │ │ └─────────┘ └─────────┘ └─────────────┘ │ │ │ │ └──────────────────────────────────────────────┘ │ │ │ │ │ │ │ ▼ │ │ │ ┌──────────────────────────────────────────────┐ │ │ │ │ Data Tier │ │ │ │ │ │ │ │ │ │ ┌────────────────┐ ┌──────────────────┐ │ │ │ │ │ │ TimescaleDB │ │ PostgreSQL │ │ │ │ │ │ │ (Time-series) │ │ (Relational + │ │ │ │ │ │ │ │ │ Staging DB) │ │ │ │ │ │ │ Read replicas │ │ Read replicas │ │ │ │ │ │ │ for analytics │ │ for reporting │ │ │ │ │ │ └────────────────┘ └──────────────────┘ │ │ │ │ │ ┌────────────────┐ ┌──────────────────┐ │ │ │ │ │ │ Redis Cache │ │ Azure Blob / │ │ │ │ │ │ │ (Session, │ │ S3 (Photos, │ │ │ │ │ │ │ Real-time) │ │ Signatures) │ │ │ │ │ │ └────────────────┘ └──────────────────┘ │ │ │ │ └──────────────────────────────────────────────┘ │ │ │ │ │ │ ┌──────────────────────────────────────────────┐ │ │ │ │ Express Powerr On-Premise │ │ │ │ │ (Klang Depot / HQ) │ │ │ │ │ │ │ │ │ │ ┌──────────────────┐ ┌──────────────┐ │ │ │ │ │ │ SQL Account ERP │ │ Staging DB │ │ │ │ │ │ │ (SQL Server) │ │ Read Replica │ │ │ │ │ │ │ │ │ (poll only) │ │ │ │ │ │ │ DO, Ledger, │ │ │ │ │ │ │ │ │ e-Invoice, SST │ │ │ │ │ │ │ │ └──────────────────┘ └──────────────┘ │ │ │ │ │ ┌──────────────────┐ │ │ │ │ │ │ Admin Workstations│ │ │ │ │ │ │ (Finance/ │ │ │ │ │ │ │ Dispatch access) │ │ │ │ │ │ └──────────────────┘ │ │ │ │ └──────────────────────────────────────────────┘ │ │ │ │ │ │ VPN / Azure ExpressRoute ◄────────────► On-Prem │ │ │ (Private connectivity for staging sync) │ │ └──────────────────────────────────────────────────────────────────────────────────────────────┘

## 7.2 Environment Breakdown

| Environment | Purpose | DB Sizing | Scaling |
| --- | --- | --- | --- |
| Production | Live fleet operations | TimescaleDB: 4 vCPU, 16 GB RAM, 500 GB SSD PostgreSQL: 4 vCPU, 16 GB, 200 GB | Auto-scale (2–8 API pods), DB multi-AZ |
| Staging | Integration testing with ERP | TimescaleDB: 2 vCPU, 8 GB PostgreSQL: 2 vCPU, 8 GB | Fixed (2 API pods), single-AZ |
| Dev/QA | Feature development & QA testing | Shared small instances | Fixed (1 API pod) |

## 7.3 Disaster Recovery

Production databases are deployed in a multi-AZ configuration with automated backups (Point-in-Time Recovery, 35-day retention). Cross-region replication to Malaysia (if Azure Malaysia region is available) or a secondary region is configured for DR. RPO: 5 minutes, RTO: 1 hour.

# 8. Security Architecture

## 8.1 Authentication & Authorisation

The system uses OAuth 2.0 with the following flows:

| Flow | Used By | Token Type |
| --- | --- | --- |
| Authorization Code + PKCE | Driver Mobile App, Admin Web Portal (user-facing) | JWT (access) + Refresh Token |
| Client Credentials | ERP Sync Bridge, webhook consumers (machine-to-machine) | JWT (access, no refresh) |

JWTs are signed with RS256. The public key is exposed at /.well-known/jwks.json for third-party verification. Access tokens expire after 60 minutes; refresh tokens after 30 days.

## 8.2 Role-Based Access Control (RBAC)

| Role | Scope | Elevated Permissions |
| --- | --- | --- |
| Super Admin | Full system access | User management, ERP sync retry, system config |
| Fleet Manager | All fleet, fuel, dispatch, SLA | Geofence CRUD, alert configuration |
| Dispatcher | Dispatch, job management | Job creation, vehicle/crew assignment |
| Driver / Engine Driver | Own jobs only (mobile app) | POD submission, job accept/reject |
| Chargeman | Own jobs only (mobile app) | POD co-sign, generator handover certification |
| Finance | ERP sync, fuel logs (read) | Manual retry of staging records, export reports |
| Viewer | Dashboards and reports only | No write operations |

## 8.3 Data Encryption

In Transit: All API traffic requires TLS 1.2+. Device telemetry uses MQTTS (TLS 1.2). VPN/ExpressRoute between cloud and Express Powerr on-premise network.

At Rest: All databases use transparent data encryption (TDE). File store (photos, signatures) is encrypted with AES-256 using KMS-managed keys rotated every 90 days.

Secrets: API keys, database credentials, and webhook secrets are stored in Azure Key Vault / AWS Secrets Manager with automatic rotation.

## 8.4 Audit Logging

All write operations across the system are logged to an immutable audit trail including: actor ID, action, resource, timestamp, old/new values (for critical fields), and IP address. Audit logs are retained for 7 years to comply with LHDN and SST record-keeping requirements.

## 8.5 LHDN & e-Invoice Compliance

The Smart Fleet system does not directly submit e-Invoices to LHDN. Instead, it generates validated staging records that the existing SQL Account ERP consumes. The ERP's existing e-Invoice module handles the LHDN MyInvois submission. The staging schema includes fields required for MyInvois compliance (buyer TIN, SST registration no., item classification code) so that the finance team does not need to re-key data.

## 8.6 Security Controls Summary

| Control | Implementation |
| --- | --- |
| API Rate Limiting | 100 req/min per user, 1000 req/min per client credential. Burst allowance of 20%. |
| Webhook Signing | HMAC-SHA256 signature in X-Webhook-Signature header. Secret per consumer. |
| Device Authentication | Each telematics gateway has a unique API key + device certificate (mTLS). |
| SQL Injection Prevention | Parameterised queries only. Staging DB runs with restricted permissions (INSERT only on staging tables). |
| CORS | Restricted to Express Powerr's admin portal domain and authorised mobile app origins. |
| DDoS Protection | Azure Front Door / AWS CloudFront with WAF rules blocking malicious traffic. |
| Penetration Testing | Annual third-party pentest required before go-live. |

# Appendix A — Glossary of Terms

| Term | Definition |
| --- | --- |
| BLE | Bluetooth Low Energy — used for beacon-based generator-to-vehicle binding |
| CAN-bus / J1939 | Controller Area Network protocol — standard for heavy-duty vehicle telemetry |
| Capacitance Probe | Submersible fuel level sensor that measures dielectric change between electrodes |
| Chargeman | Certified electrical supervisor responsible for generator connection and safety on-site |
| DO (Delivery Order) | Commercial document confirming delivery of goods/services — triggers invoicing |
| Engine Driver | Operator of the generator unit, responsible for fuel, runtime, and basic maintenance |
| ERP | Enterprise Resource Planning — Express Powerr uses SQL Account for finance and DOs |
| Geofence | Virtual geographic boundary that triggers events when a vehicle enters/exits |
| LHDN | Lembaga Hasil Dalam Negeri Malaysia — Malaysian tax authority requiring e-Invoice compliance |
| Low-Loader | Low-bed trailer for transporting heavy generator units |
| mTLS | Mutual TLS — two-way certificate-based authentication for IoT device connections |
| OBD-II | On-Board Diagnostics — standard vehicle interface for engine telemetry |
| POD | Proof of Delivery — digital signature + photo evidence of generator placement |
| Prime Mover | Heavy tractor unit for towing low-loaders carrying generators |
| SESB | Sabah Electricity Sdn Bhd — utility client in East Malaysia |
| SLA | Service Level Agreement — contractual response time guarantees for TNB/SESB |
| SQL Account | Express Powerr's existing ERP system for finance, DOs, and e-Invoicing |
| SST | Sales and Service Tax — Malaysian consumption tax reported via ERP |
| Staging DB | Intermediate database that buffers data between Fleet Cloud and live ERP |
| TNB | Tenaga Nasional Berhad — Malaysia's national electricity utility |

— End of Document —